附录  核心代码

## 附1. 下位机数据结构定义（Modules.h）

#ifndef __MODULES_H_

#define __MODULES_H_

#include "stm32f10x.h"

#include "dht11.h"

#include "adcx.h"

#include "rtc.h"

#include "hx711.h"

#include "HW.h"

#include "water.h"

typedef struct

{

uint8_t humi;      // 湿度

uint8_t temp;      // 温度

float weight;      // 重量(g)

_calendar_obj calendarData;  // RTC日历

uint16_t water;    // 水位(%)

} SensorModules;

// 定时喂食项

typedef struct

{

_calendar_obj time;       // 定时时间

uint16_t feed_weight;     // 喂食量(g)

uint8_t enabled;          // 启用标志

} ScheduleItem;

typedef struct

{

float weight_Value;               // 称重阈值(g)

uint16_t water_Value;             // 水位阈值(%)

ScheduleItem schedule[3];          // 3个定时

} SensorThresholdValue;

typedef struct

{

uint8_t Food_Flag;       // 喂食请求

uint8_t Now_Food_Flag;   // 正在喂食

uint8_t Time_Food_Flag;  // 定时编号(1-3)

uint8_t Water_Flag;      // 水泵请求

uint8_t Humin_Flag;      // 红外检测

uint8_t Beep_Flag;       // 蜂鸣器

} DriveModules;

extern SensorModules sensorData;

extern SensorThresholdValue Sensorthreshold;

extern DriveModules driveData;

void SensorScan(void);

#endif

## 附2. HX711称重算法（hx711.c）

#include "hx711.h"

#include "delay.h"

#define MEDIAN_LEN  5

#define MEDIAN      3

float pi_weight;                    // 皮重

float hx711_xishu = 12.37363426011; // 校准系数

// 读取HX711 24位AD数据（含超时保护防止传感器断线死锁）

unsigned long HX711_GetData(void)

{

unsigned long Count;

unsigned char i;

volatile uint32_t timeout = 0;

HX711_SCK_L; delay_us(1); Count = 0;

while (HX711_DT) {

if (++timeout > 200000) return 0;

}

for (i = 0; i < 24; i++) {

HX711_SCK_H; delay_us(1);

Count = Count << 1;

HX711_SCK_L; delay_us(1);

if (HX711_DT) Count++;

}

HX711_SCK_H; delay_us(1);

Count = Count ^ 0x800000;

HX711_SCK_L; delay_us(1);

return Count;

}

// 获取当前重量（5次采样取均值 → 减皮重 → 线性换算）

float Get_Weight(float pi_weight)

{

uint32_t hx711_data;

float weight, get, sum = 0;

uint8_t i;

for (i = 0; i < 5; i++) {

hx711_data = HX711_GetData();

get = ((float)hx711_data / 8388607.0) * 1000 * hx711_xishu;

sum += get;

}

get = sum / 5.0f;

weight = get - pi_weight;

return weight;

}

// 获取皮重（5点中值滤波 → 插入排序）

float Get_Tare(void)

{

uint32_t hx711_dat, buffer[5];

uint8_t i, j, medleng = 0;

for (i = 0; i < 5; i++) {

hx711_dat = HX711_GetData();

if (medleng == 0) {

buffer[0] = hx711_dat; medleng = 1;

} else {

for (j = 0; j < medleng; j++) {

if (buffer[j] > hx711_dat) {

uint32_t xd = hx711_dat;

hx711_dat = buffer[j]; buffer[j] = xd;

}

}

buffer[medleng] = hx711_dat; medleng++;

}

if (medleng >= 5) {

hx711_dat = buffer[2];  // 取中值

medleng = 0;

}

}

pi_weight = ((float)hx711_dat / 8388607.0) * 1000 * hx711_xishu;

return pi_weight;

}

## 附3. 步进电机非阻塞控制（stepmotor.c）

#include "stepmotor.h"

// 电机状态机变量

volatile uint8_t  motor_running = 0;

volatile int16_t  motor_steps = 0;

volatile uint8_t  motor_step_idx = 0;

volatile uint8_t  motor_dir = 0;

volatile uint8_t  motor_continuous = 0;

// 八拍相序表（前转/反转）

static const uint8_t motor_seq_fwd[8] = {0x01,0x03,0x02,0x06,

0x04,0x0C,0x08,0x09};

static const uint8_t motor_seq_rev[8] = {0x09,0x08,0x0C,0x04,

0x06,0x02,0x03,0x01};

// 按角度启动（4096步=360度）

void MOTOR_START(uint8_t dir, uint16_t angle)

{

motor_steps = (int16_t)((uint32_t)4096 * angle / 360);

motor_dir = dir;

motor_step_idx = 0;

motor_continuous = 0;

motor_running = 1;

}

// 持续转动模式

void MOTOR_START_CONTINUOUS(uint8_t dir)

{

motor_dir = dir;

motor_step_idx = 0;

motor_continuous = 1;

motor_running = 1;

}

// 紧急停止

void MOTOR_ABORT(void)

{

motor_running = 0;

motor_steps = 0;

MOTOR_STOP();

}

// 单步推进——在TIM2中断中以1ms间隔调用

void MOTOR_StepTick(void)

{

if (!motor_running) return;

const uint8_t *seq = motor_dir ? motor_seq_rev : motor_seq_fwd;

uint8_t phase = seq[motor_step_idx];

if (phase & 0x01) MOTOR_A_HIGH; else MOTOR_A_LOW;

if (phase & 0x02) MOTOR_B_HIGH; else MOTOR_B_LOW;

if (phase & 0x04) MOTOR_C_HIGH; else MOTOR_C_LOW;

if (phase & 0x08) MOTOR_D_HIGH; else MOTOR_D_LOW;

if (!motor_continuous) motor_steps--;

motor_step_idx = (motor_step_idx + 1) & 7;

if (!motor_continuous && motor_steps <= 0) {

motor_running = 0; MOTOR_STOP();

}

}

## 附4. 增量式智能喂食与防卡粮算法（Control_Manager）

// 增量称重：记录启动瞬间的初始重量

static float start_weight = 0;

// 防卡粮参数：正转1800度(5圈) → 反转360度(1圈) → 循环

#define FEED_FORWARD_ANGLE  1800

#define FEED_REVERSE_ANGLE  360

static uint8_t anti_jam_state = 0;

void Control_Manager(void)

{

// ==== 增量称重判定：当前重量 >= 启动重量 + 目标增量 ====

if (driveData.Food_Flag) {

float target;

if (driveData.Time_Food_Flag >= 1 && driveData.Time_Food_Flag <= 3)

target = Sensorthreshold.schedule[

driveData.Time_Food_Flag - 1].feed_weight;

else

target = Sensorthreshold.weight_Value;

if (sensorData.weight >= start_weight + target) {

driveData.Food_Flag = 0;

driveData.Time_Food_Flag = 0;

driveData.Now_Food_Flag = 0;

MOTOR_ABORT();  // 增量达到 → 停止

}

}

// ==== 喂食电机控制 + 防卡粮 ====

if (driveData.Food_Flag) {

if (driveData.Now_Food_Flag == 0) {

// 首次启动：记录初始重量，正转5圈

driveData.Now_Food_Flag = 1;

start_weight = sensorData.weight;

anti_jam_state = 0;

MOTOR_START(0, FEED_FORWARD_ANGLE);

} else {

// 电机停转时自动切换方向（防卡粮）

if (motor_running == 0) {

if (anti_jam_state == 0) {

anti_jam_state = 1;

MOTOR_START(1, FEED_REVERSE_ANGLE);  // 正转完→反转1圈

} else {

anti_jam_state = 0;

MOTOR_START(0, FEED_FORWARD_ANGLE);  // 反转完→正转5圈

}

}

}

} else {

anti_jam_state = 0;

if (driveData.Now_Food_Flag == 1) {

driveData.Now_Food_Flag = 0;

MOTOR_ABORT();

}

}

// ==== 水泵控制 ====

if (driveData.Water_Flag) RELAY_ON; else RELAY_OFF;

}

## 附5. 自动补水联动控制（AutoControl）

void AutoControl(void)

{

// 3个定时器触发判定

for (int i = 0; i < 3; i++) {

if ((sensorData.calendarData.hour ==

Sensorthreshold.schedule[i].time.hour &&

sensorData.calendarData.min ==

Sensorthreshold.schedule[i].time.min)

&& !food_flag[i] &&

Sensorthreshold.schedule[i].enabled)

{

driveData.Food_Flag = 1;

food_flag[i] = 1;

driveData.Time_Food_Flag = i + 1;

driveData.Beep_Flag = 1;

}

}

// 跨日重置定时触发标志

if (date != sensorData.calendarData.w_date) {

date = sensorData.calendarData.w_date;

for (int i = 0; i < 3; i++) food_flag[i] = 0;

}

// 自动补水：水位不足 + 红外确认有宠物靠近

if ((sensorData.water < Sensorthreshold.water_Value) &&

(driveData.Humin_Flag == 1))

driveData.Water_Flag = 1;

else

driveData.Water_Flag = 0;

}

## 附6. ESP8266指令解析与Flash读写

// === WiFi_Key(): APP指令解析（来自BlueCmd接收缓冲区） ===

void WiFi_Key(void)

{

if (BlueCmd("KA",20)) KeyNum = KEY_1;  // 切换模式

if (BlueCmd("KB",20)) KeyNum = KEY_2;  // 移动光标

if (BlueCmd("KC",20)) KeyNum = KEY_3;  // 开启

if (BlueCmd("KD",20)) KeyNum = KEY_4;  // 关闭

// 直通指令（不依赖光标）

if (BlueCmd("MW0",20)) driveData.Water_Flag = 1;

if (BlueCmd("MW1",20)) driveData.Water_Flag = 0;

if (BlueCmd("MF0",20) && driveData.Now_Food_Flag == 0)

driveData.Food_Flag = 1;

if (BlueCmd("MF1",20)) {

driveData.Food_Flag = 0;

driveData.Now_Food_Flag = 0;

MOTOR_ABORT();

}

// SW+3位数字 → 水位阈值

{   char *sw = strstr((const char *)Res, "SW");

if (sw && sw[2]>='0' && sw[2]<='9' && sw[3]>='0'

&& sw[3]<='9' && sw[4]>='0' && sw[4]<='9') {

int val = (sw[2]-'0')*100 + (sw[3]-'0')*10 + (sw[4]-'0');

if (val >= 0 && val <= 100) {

Sensorthreshold.water_Value = val;

Flag_Writer();  // 写Flash

}

}

}

// SF+4位数字 → 称重阈值

{   char *sf = strstr((const char *)Res, "SF");

if (sf) {

int val = (sf[2]-'0')*1000 + (sf[3]-'0')*100

+ (sf[4]-'0')*10 + (sf[5]-'0');

if (val >= 0 && val <= 9999) {

Sensorthreshold.weight_Value = val;

Flag_Writer();

}

}

}

// STnHHMM → 定时时间 / SDnXXXX → 喂食量 / SEn0/1 → 启用

// TIMEHHMMSS → RTC时间同步

}

// === Flag_Writer(): 参数写Flash（0x0800F000）===

#define FLASH_START_ADDR  0x0800F000

void Flag_Writer(void)

{

u16 dat[14];

dat[0] = Sensorthreshold.schedule[0].time.hour;

dat[1] = Sensorthreshold.schedule[0].time.min;

dat[2] = Sensorthreshold.schedule[1].time.hour;

dat[3] = Sensorthreshold.schedule[1].time.min;

dat[4] = Sensorthreshold.schedule[2].time.hour;

dat[5] = Sensorthreshold.schedule[2].time.min;

dat[6] = (u16)Sensorthreshold.weight_Value;

dat[7] = Sensorthreshold.water_Value;

dat[8]  = (u16)Sensorthreshold.schedule[0].feed_weight;

dat[9]  = (u16)Sensorthreshold.schedule[1].feed_weight;

dat[10] = (u16)Sensorthreshold.schedule[2].feed_weight;

dat[11] = Sensorthreshold.schedule[0].enabled;

dat[12] = Sensorthreshold.schedule[1].enabled;

dat[13] = Sensorthreshold.schedule[2].enabled;

FLASH_W(FLASH_START_ADDR, dat, 14);

}

// === Flash底层驱动（Flash.c）===

void FLASH_W(u32 add, u16 *dat, u8 count)

{

u8 i;

FLASH_Unlock();

FLASH_ErasePage(add);

for (i = 0; i < count; i++)

FLASH_ProgramHalfWord(add + i * 2, dat[i]);

FLASH_Lock();

}

u16 FLASH_R(u32 add)

{

return *(volatile u16 *)add;

}

// === Flash初始化（读取 + 合法性校验 + 默认值）===

void Flash_Time_Init(void)

{

Sensorthreshold.schedule[0].time.hour = FLASH_R(FLASH_START_ADDR);

Sensorthreshold.schedule[0].time.min  = FLASH_R(FLASH_START_ADDR + 2);

// ...读取所有参数...

if (Sensorthreshold.schedule[0].time.hour > 24 || ...) {

u16 def[14] = {12,1, 12,2, 12,3, 200, 80,

50, 100, 150, 1, 1, 1};

FLASH_W(FLASH_START_ADDR, def, 14);

}

}

## 附7. TIM2定时器中断（tim2.c）

// TIM2初始化: 72MHz / (9+1) / (7198+1) = 1ms

void TIM2_Init(void)

{

TIM_TimeBaseInitTypeDef TIM_TimeBaseStructure;

NVIC_InitTypeDef NVIC_InitStructure;

RCC_APB1PeriphClockCmd(RCC_APB1Periph_TIM2, ENABLE);

TIM_TimeBaseStructure.TIM_Period = 9;

TIM_TimeBaseStructure.TIM_Prescaler = 7198;

TIM_TimeBaseStructure.TIM_ClockDivision = 0;

TIM_TimeBaseStructure.TIM_CounterMode = TIM_CounterMode_Up;

TIM_TimeBaseInit(TIM2, &TIM_TimeBaseStructure);

TIM_ITConfig(TIM2, TIM_IT_Update, ENABLE);

NVIC_InitStructure.NVIC_IRQChannel = TIM2_IRQn;

NVIC_InitStructure.NVIC_IRQChannelPreemptionPriority = 2;

NVIC_InitStructure.NVIC_IRQChannelSubPriority = 0;

NVIC_InitStructure.NVIC_IRQChannelCmd = ENABLE;

NVIC_Init(&NVIC_InitStructure);

TIM_Cmd(TIM2, ENABLE);

}

// 1ms中断服务：系统时钟 + 按键扫描 + 步进电机

void TIM2_IRQHandler(void)

{

if (TIM_GetITStatus(TIM2, TIM_IT_Update) != RESET) {

sys_tick_ms++;

Key_scan();       // 按键扫描

MOTOR_StepTick(); // 步进电机单步推进

TIM_ClearITPendingBit(TIM2, TIM_IT_Update);

}

}

## 附8. IWDG独立看门狗（iwdg.c）

// IWDG初始化：预分频64，重载1875 → 约3秒超时

// 超时 = (64/40kHz) * 1875 ≈ 3.0s

void IWDG_Init(u8 prer, u16 rlr)

{

IWDG_WriteAccessCmd(IWDG_WriteAccess_Enable);

IWDG_SetPrescaler(prer);

IWDG_SetReload(rlr);

IWDG_ReloadCounter();

IWDG_Enable();

}

void IWDG_Feed(void)

{

IWDG_ReloadCounter();  // 0xAAAA

}

## 附9. 上位机APP TCP通信与数据帧解析（MainActivity.kt）

// 数据类

data class ScheduleItem(

val hour: Int = 7,

val minute: Int = 0,

val feedWeight: Int = 50,

val enabled: Boolean = true

)

data class DeviceData(

val temperature: String = "--°C",

val humidity: String = "--%",

val weight: String = "-- g",

val waterLevel: String = "--%",

val isConnected: Boolean = false,

val isReconnecting: Boolean = false,

val isAutoMode: Boolean = false,

val isFeeding: Boolean = false,

val isWaterPumpOn: Boolean = false,

val weightThreshold: Int = 200,

val waterThreshold: Int = 50,

val schedule1: ScheduleItem = ScheduleItem(7,0,50,true),

val schedule2: ScheduleItem = ScheduleItem(12,30,100,true),

val schedule3: ScheduleItem = ScheduleItem(19,0,150,true),

)

// TCP通信客户端

class TcpClient(private val scope: CoroutineScope) {

private var socket: Socket? = null

private var reader: BufferedReader? = null

private var writer: PrintWriter? = null

private val _deviceData = MutableStateFlow(DeviceData())

val deviceData: StateFlow<DeviceData> = _deviceData.asStateFlow()

private var host = "192.168.4.1"

private val port = 5000

// 正则解析数据帧

private val dataPattern = Pattern.compile(

"^T(\d+)H(\d+)W(\d+)L(\d+)M(\d)F(\d)P(\d)" +

"WT(\d+)FT(\d+)" +

"T1(\d{2})(\d{2})F1(\d+)E(\d)" +

"T2(\d{2})(\d{2})F2(\d+)E(\d)" +

"T3(\d{2})(\d{2})F3(\d+)E(\d).*"

)

// 连接服务器（10秒超时）

suspend fun connect(): Boolean = withContext(Dispatchers.IO) {

try {

socket = Socket()

socket!!.connect(InetSocketAddress(host, port), 5000)

socket!!.soTimeout = 10000

reader = BufferedReader(InputStreamReader(socket!!.getInputStream()))

writer = PrintWriter(socket!!.getOutputStream(), true)

_deviceData.update { it.copy(isConnected = true) }

startReceiving()

syncTimeToDevice()

true

} catch (e: Exception) {

startAutoReconnect()

false

}

}

// 持续接收数据帧

private fun startReceiving() {

receiveJob = scope.launch(Dispatchers.IO) {

while (isActive && socket?.isConnected == true) {

val line = reader?.readLine() ?: break

if (line.trim().startsWith("T")) parseData(line.trim())

}

}

}

// 正则解析（含范围校验，异常保留上次有效值）

private fun parseData(data: String) {

val matcher = dataPattern.matcher(data)

if (!matcher.matches()) return

val temp = matcher.group(1)?.toIntOrNull() ?: lastValidTemp

val humi = matcher.group(2)?.toIntOrNull() ?: lastValidHumi

// ...解析所有字段并更新_deviceData

}

// 指数退避自动重连

private fun startAutoReconnect() {

reconnectJob = scope.launch(Dispatchers.IO) {

var delayMs = 1000L

while (isActive) {

delay(delayMs)

try {

socket = Socket()

socket!!.connect(InetSocketAddress(host, port), 5000)

_deviceData.update {

it.copy(isConnected = true, isReconnecting = false)

}

startReceiving()

syncTimeToDevice()

return@launch

} catch (e: Exception) {

delayMs = (delayMs * 2).coerceAtMost(30000L)

}

}

}

}

// 指令封装

fun applyWaterThreshold(value: Int) {

scope.launch(Dispatchers.IO) {

sendCommand(String.format("SW%03d", value))

}

}

fun applyWeightThreshold(value: Int) {

scope.launch(Dispatchers.IO) {

sendCommand(String.format("SF%04d", value))

}

}

}